CESAR LEONEL CHAMALE SICAN	-	201700634 		GRUPO: 8

La X significa el [Número de Grupo + 2 Últimos Números de su carnet]

|  |  |  |  | 201700634 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| x | = | 8 | + | 3 | + | 4 |
| x | = | 15 |  |  |  |  |

| TOPOLOGIA | DIRECCION IP | GATEWAY |
| --- | --- | --- |
| 1 | 192.168.151.10/24 | 192.168.151.1 |
| 2 | 192.168.151.20/24 | 192.168.151.1 |
| 3 | 192.168.152.10/24 | 192.168.152.1 |
| 4 | 192.168.152.20/24 | 192.168.152.1 |
| 5 | 192.168.153.10/24 | 192.168.153.1 |
| 6 | 192.168.153.20/24 | 192.168.153.1 |

| TOPOLOGIA | DIRECCION DE RED | PRIMERA DIRECCION ASIGNABLE | GATEWAY |
| --- | --- | --- | --- |
| R1-R2 | 172.151.0.0/16 | 172.151.0.1 | N.A. |
| R1-R3 | 172.152.0.0/16 | 172.152.0.1 | N.A. |
| R3-R2 | 172.153.0.0/16 | 172.153.0.1 | N.A. |

CONFIGURAMOS EL PUERTO EN LOS SWITCHES PARA PONERLOS EN MODO TRUNCAL

CONFIGURAMOS EL MODO DE FASTHETERNET Y PUERTOS SERIAL EN CADA ROUTER

- el Router con Seriales en el Slot 1

el Router con Seriales en el Slot 2

- el Router con Seriales en el Slot 3

conectar los Routers de esta manera:

- R1 – R2   S1/0 – S2/0

- R1 – R3  S1/1 – S3/0

- R2 – R3  S2/1 -S3-1

CONFIGURAMOS TODOS LA CONECCIONES DE LA SIGUIENTE MANERA:

COMANDOS:

VPCs

VPC1

ip 192.168.151.10/24 192.168.151.1

save

VPC 2

ip 192.168.151.20/24 192.168.151.1

save

VPC 3

ip 192.168.152.10/24 192.168.152.1

save

VPC 4

ip 192.168.152.20/24 192.168.152.1

save

VPC 5

ip 192.168.153.10/24 192.168.153.1

save

VPC 6

ip 192.168.153.20/24 192.168.153.1

save

Routers (Interfaces de comunicación con SWITCHES)

R1

configure terminal

int f0/0

ip address 192.168.153.1 255.255.255.0

no shutdown

exit

R2

configure terminal

int f0/0

ip address 192.168.152.1 255.255.255.0

no shutdown

exit

R3

configure terminal

int f0/0

ip address 192.168.151.1 255.255.255.0

no shutdown

exit

Routers (Interfaces de comunicación con VPCs)

R1 - R2

configure terminal

int s1/0

ip address 172.151.0.1 255.255.0.0

no shutdown

exit

R2 - R1

configure terminal

int s2/0

ip address 172.151.0.2 255.255.0.0

no shutdown

exit

R1 - R3

configure terminal

int s1/1

ip address 172.152.0.1 255.255.0.0

no shutdown

exit

R3 - R1

configure terminal

int s3/0

ip address 172.152.0.2 255.255.0.0

no shutdown

exit

R3 - R2

configure terminal

int s3/1

ip address 172.153.0.1 255.255.0.0

no shutdown

exit

R2 - R3

configure terminal

int s2/1

ip address 172.153.0.2 255.255.0.0

no shutdown

exit

PINGS

1

ping 192.168.151.10

2

ping 192.168.151.20

3

ping 192.168.152.10

4

ping 192.168.152.20

5

ping 192.168.153.10

6

ping 192.168.153.20

Routers (Enrutamiento estático)

R1 - R2

conf t

ip route 192.168.152.0 255.255.255.0 172.151.0.2

exit

R2 - R1

conf t

ip route 192.168.153.0 255.255.255.0 172.151.0.1

exit

R1 - R3

conf t

ip route 192.168.151.0 255.255.255.0 172.152.0.2

exit

R3 - R1

conf t

ip route 192.168.153.0 255.255.255.0 172.152.0.1

exit

R3 - R2

conf t

ip route 192.168.152.0 255.255.255.0 172.153.0.2

exit

R2 - R3

conf t

ip route 192.168.151.0 255.255.255.0 172.153.0.1

exit

ROUTER 1

- sh ip int br

- sh ip ro

ROUTER 2

- sh ip int br

- sh ip ro

ROUTER 3

- sh ip int br

- sh ip ro

PINGS